# Gait Control of the Lizard V1 Robot

A Self-Contained Mathematical Reference for the MuJoCo Simulation

Lizard Robot Project

2026

Table of Contents

# Preface

This document is a complete, self-contained description of the locomotion controllers used in the Lizard V1 MuJoCo simulation. It is written so that a reader with first-year calculus and trigonometry — and no prior knowledge of this project — can understand exactly how every joint command is generated.

Each control law is presented as a set of equations, followed by the meaning of every symbol, the reason the equation has the form it does, and the numerical values currently used. Two laws are described:

- the Sine law, a smooth traveling-wave gait that we optimized for fast, straight walking; and

- the Hardware law, a port of the real robot’s controller, in which the body holds a standing “C” posture that flips side in time with a diagonal trot of the legs.

# Chapter 1. The Robot and Its Notation

## 1.1 Degrees of freedom

The robot has a central body with an articulated spine and four single-joint legs, for a total of seven actuated revolute joints:

| Group | Joints | Axis | Role |
| --- | --- | --- | --- |
| Spine | Front, Back, Tail | vertical (yaw) | lateral body bending |
| Front legs | FR, FL | longitudinal | press / lift the front feet |
| Rear legs | HR, HL | longitudinal | press / lift the rear feet |

Each joint is driven by a position-controlled Dynamixel XL430 servo, whose commanded angle we write as qj(t) for joint j at time t (in radians).

## 1.2 Reference frame

We use a right-handed world frame fixed to the ground at the spawn point: +x is forward, +z is up, and +y is to the left. The robot’s net progress is measured by the forward displacement of its body centre, Δx=x(T)−x(Ts), evaluated after the start-up transient (Section 2.2).

## 1.3 Index sets and labels

Let the ordered list of spine joints be S=(Front,Back,Tail) with Ns=3 members, and assign each spine joint an integer backbone index:

i(Front)=0,  i(Back)=1,  i(Tail)=2.

The four legs are grouped two ways that we use repeatedly:

right={FR,HR},  left={FL,HL},

front={FR,FL},  hind={HR,HL}.

Each leg ℓ also has a girdle g(ℓ)∈S — the spine joint it is attached near (Front for the front legs, Back for the rear legs).

# Chapter 2. Common Control Architecture

Both laws are open-loop: the controller is a pure function of time. At every simulation step it computes a target angle qj(t) for each joint, and the MuJoCo position actuator applies a torque proportional to the tracking error, saturated at the servo stall torque.

## 2.1 The general form

Every joint command has the same three-part structure,

qj(t)=θj+r(t) wj(t),

where θj is the initial angle (or offset) of joint j — the posture the joint holds before the gait starts and the value about which it oscillates; wj(t) is the time-varying oscillation produced by the chosen control law (Chapters 3–4); and r(t)∈[0,1] is a dimensionless start-up ramp shared by all joints.

## 2.2 The start-up ramp

To avoid a violent jump when the gait switches on, the oscillation amplitude is faded in with a raised-cosine ramp over a settling time Ts followed by a ramp time Tr:

r(t)=0,t≤Ts,12 1−cos​π t−TsTr,Ts<t<Ts+Tr,1,t≥Ts+Tr.

During the settling phase (t≤Ts) every joint simply holds its offset θj, letting the robot settle onto the ground. The ramp is C1-continuous, so both r and its derivative start at zero and there is no acceleration discontinuity. Default values are Ts=1.0 s and Tr=1.5 s.

# Chapter 3. The Sine Law (Traveling Wave)

The sine law treats the body as a continuous medium carrying a sinusoidal traveling wave, with the legs phase-locked to it.

## 3.1 Spine: a traveling wave

For a spine joint with backbone index i,

qi(t)=θi+r(t) b+r(t) As sin​(2πf t+φi),

with the spatial phase

φi=2π k cNs−1 i.

The symbols are:

| Symbol | Name | Meaning |
| --- | --- | --- |
| As | spine.amplitude | peak bending angle of a spine joint (rad) |
| f | gait.frequency | temporal frequency of oscillation (Hz) |
| k | spine.wavenumber | number of full sine waves along the body |
| c | gait.wave_speed | spatial phase-propagation multiplier |
| b | spine.turn_trim | constant yaw bias used to steer (rad) |
| θi | offset | per-joint initial angle (rad) |

Why this form. The term 2πft makes every joint oscillate at the same frequency f. The spatial term φi adds a fixed phase lag between consecutive joints; because i runs from 0 to Ns−1, the head-to-tail phase difference is φNs−1−φ0=2πkc, i.e. exactly kc full waves span the body. A nonzero lag makes the crest travel from head to tail, which propels the robot; the steering bias b shifts the whole body slightly to one side to trim a curved path into a straight one.

## 3.2 Legs: antisymmetric sine

Each leg oscillates at the same frequency, locked to the spatial phase of its girdle, with the left legs exactly opposite the right legs:

qℓ(t)=θℓ+r(t) σℓ Aℓ sin​(2πf t+φg(ℓ)+2πψ),

where the side sign is

σℓ=+1,ℓ∈right={FR,HR},−1,ℓ∈left={FL,HL}.

Here Aℓ is legs.amplitude and ψ is the single leg-versus-body phase offset legs.phase_offset (in cycles, hence the factor 2π). The sign σℓ makes the left and right legs mirror one another, so their lateral reaction forces tend to cancel and the net motion is forward.

# Chapter 4. The Hardware Law (Standing C-Wave + Diagonal Trot)

This law reproduces the real robot’s controller. The body no longer carries a traveling wave; instead it bends into a single C shape that reverses each half-cycle, while the legs run a duty-cycled diagonal trot locked to that bending.

## 4.1 Spine: a standing C-wave

Every spine joint shares one and the same sinusoid — there is no per-segment phase lag:

qi(t)=θi+r(t) Ab mi sin​(2πfℓ t+λ).

| Symbol | Name | Meaning |
| --- | --- | --- |
| Ab | body_amp | C-bend amplitude (rad) |
| fℓ | leg_freq | gait frequency; the body bends at the leg frequency (Hz) |
| λ | body_lock | phase that times the bend against foot contact (rad) |
| mi | body_shape | per-joint shape weight, mi∈{+1,−1} |

Because the time argument 2πfℓt+λ is identical for all joints, the body is a standing wave: at any instant every spine joint is displaced by the same signed factor sin(2πfℓt+λ), scaled by its shape weight mi. As that factor swings between +1 and −1, the body curls one way, straightens, and curls the other way.

## 4.2 Why the C requires opposite signs (the C-vs-S rule)

A natural guess is to set all shape weights equal, mFront=mBack=mTail=+1. This produces an S, not a C. The reason is geometric. The Front section extends in +x from the body centre, while the Back and Tail sections extend in −x. All three yaw joints rotate about the same world axis −z, so a positive joint angle rotates any attached point by the same sense. Under a rotation by +α about −z, a tip pointing in +x moves toward −y, while a tip pointing in −x moves toward +y. Hence equal-sign commands send the head and the tail to opposite lateral sides (an S). To bend head and tail to the same side (a C), the rear joints must take the opposite sign of the front joint:

mFront=+1,  mBack=mTail=−1.

This is a property of the kinematic layout, not of the joint-axis polarity (all three axes share the same positive direction).

## 4.3 Legs: the duty-cycle function

A leg should press its foot down for part of the cycle (stance) and lift it for the rest (swing). This is captured by a duty function of the cycle fraction u∈[0,1) and the stance fraction d= duty:

D(u;d)=sin​πud,0≤u<d (stance: foot down),0,d≤u<1 (swing: foot neutral).

During stance the foot makes one smooth down-and-up press (the half-sine rises to 1 at u=d/2 and returns to 0); during swing it rests at the neutral posture. The fraction d is the duty factor: d=0.5 means the foot is down for half of each stride.

## 4.4 Legs: the diagonal trot

Each leg’s place in the cycle is set by adding a phase ϕlr= phi_leg for being on the right and a phase ϕfb= phi_lat for being at the rear:

ϕℓ(t)=2πfℓ t+[ ℓ∈right ] ϕlr+[ ℓ∈hind ] ϕfb,

where [ ⋅ ] equals 1 when the condition holds and 0 otherwise. The cycle fraction and the final leg command are then

uℓ(t)=ϕℓ(t)2π,  qℓ(t)=θℓ+r(t) eℓ Aℓ D​(uℓ(t); d),

where ⟨⋅⟩ denotes the fractional part, Aℓ= leg_amp, and eℓ= down_sign ∈{+1,−1} is the sign that drives each particular foot downward (it differs between legs because of mirrored mounting). With the canonical choice ϕlr=ϕfb=π, the four legs split into the two diagonal pairs of a trot:

| Leg | Added phase | Contact window |
| --- | --- | --- |
| FL (left, front) | 0 | first half |
| HR (right, hind) | π+π≡0 | first half |
| FR (right, front) | π | second half |
| HL (left, hind) | π | second half |

Thus the pair {FL,HR} presses together, then {FR,HL} presses together — the diagonal gait of a walking lizard.

## 4.5 Locking the bend to the contact

The body phase λ (body_lock) is chosen so the C-bend and the foot contacts are synchronized. At the tuned value the rule is: when the body bends left, the pair {FL,HR} is in contact; when it bends right, the pair {FR,HL} is in contact. In words, the body always bends toward the side whose front foot is planted. Shifting λ by π swaps the two correspondences (a half-cycle offset), which is exactly how the relationship was corrected during tuning.

# Chapter 5. Optimization of the Gait

The continuous parameters of either law are tuned by simulation-in-the-loop search. We run many short head-less rollouts and score each by the body’s forward progress.

## 5.1 Objectives

For raw distance we maximize the forward displacement after the settling time,

Jfwd=x(T)−x(Ts).

For a straight fast gait we penalize lateral drift Δy and net heading change Δψ (final yaw minus initial yaw):

Jstraight=Δx−wy |Δy|−wψ |Δψ|,

with weights wy=2.0 m−1 and wψ=0.4 rad−1 used in this project. A rollout that diverges (body height greater than 1 m) is assigned a large negative score so the optimizer avoids unstable regions.

## 5.2 Search

The free parameters (amplitudes, phase offsets, duty factor, initial postures, and — when allowed — frequency) are searched with a Tree-structured Parzen Estimator (Optuna). Each study is persisted to disk and is therefore resumable. When the gait identity must be preserved (the C shape and the bend-to-front-foot rule), the structural quantities mi, ϕlr, ϕfb, and eℓ are held fixed and λ is restricted to a neighbourhood of π.

# Appendix A. Symbol Table

| Symbol | YAML key | Units | Description |
| --- | --- | --- | --- |
| qj | — | rad | commanded angle of joint j |
| θj | offset.* | rad | per-joint initial angle / oscillation centre |
| r(t) | — | — | start-up ramp, from 0 to 1 |
| Ts, Tr | settle_time, ramp_time | s | settle and ramp durations |
| f | gait.frequency | Hz | sine-law temporal frequency |
| c | gait.wave_speed | — | sine-law spatial-propagation multiplier |
| As | spine.amplitude | rad | sine spine amplitude |
| k | spine.wavenumber | — | waves along the body |
| b | spine.turn_trim | rad | steering bias |
| Aℓ (sine) | legs.amplitude | rad | sine leg amplitude |
| ψ | legs.phase_offset | cycles | sine leg-vs-body phase |
| σℓ | legs.right, legs.left | — | leg side sign ±1 |
| Ab | hardware.body_amp | rad | C-bend amplitude |
| λ | hardware.body_lock | rad | body-vs-leg phase |
| mi | hardware.body_shape | — | spine shape weights (+1 front, −1 rear gives a C) |
| Aℓ (hw) | hardware.leg_amp | rad | foot-down amplitude |
| fℓ | hardware.leg_freq | Hz | hardware body and leg frequency |
| d | hardware.duty | — | stance fraction |
| ϕlr | hardware.phi_leg | rad | left/right leg phase |
| ϕfb | hardware.phi_lat | rad | front/back leg phase |
| eℓ | hardware.down_sign | — | foot-down sign ±1 |

# Appendix B. Reference Parameter Set

The hardware-law values currently in configs/gait.yaml:

| Parameter | Value |
| --- | --- |
| fℓ (leg_freq) | 0.2 Hz |
| Ab (body_amp) | 0.8 rad |
| λ (body_lock) | 2.234 rad |
| mi (body_shape) | (+1,−1,−1) for (Front, Back, Tail) |
| Aℓ (leg_amp) | 0.564 rad |
| d (duty) | 0.491 |
| ϕlr, ϕfb | π, π |
| eℓ (down_sign) | (−1,−1,+1,−1) for (FR, FL, HR, HL) |
| θℓ (offset, legs) | ∓0.165 rad |
| Ts, Tr | 1.0 s, 1.5 s |